ECSE 4770 LAB 5 REPORT

Dec. 2021

Yuchen Wang, Rin: 661960546, section 2

Partner: Yilu Zhou, Rin: 661931826, section 2

Abstract

- The purpose of this lab is to implement an Arm v4 CPU by Verilog and run it on the FPGA board. The basic framework of the Arm v4 CPU is provided on LMS, student need to implement serval instructions to fulfill the requirement of the lab, and run the provided instruction set of this lab. Here the missing functions in the CPU are EOR, BIC, MVN, TST, TEQ, CMP, CMN, and BL.

- After implementing these instructions, student needs to first simulate the CPU in Quartus and check the simulation result. Then run it on the FPGA board. The hex display on the board is used to verify if the instruction is correctly operated.

Overall design description

- Most code of the lab is provided. It is an Arm v4 CPU contains 16 registers, processing the instruction data and operate the instruction like common CPUs. There is no big change in the overall design.

- Students here only need to implement the ALU and data path in Verilog file. The following sections will include students implementation of each part.

- To make the CPU work, The provided file will read the instructions from a txt file provided on LMS. Then the CPU process the instructions. The result will printed on the hex display of the FPGA board. The left 2 bit is for memory address, right 2 bit for data written to memory. Which can be used to verify STR operations. The mid 2 bit is for program counter to verify the machine is running the instruction is correct order.

Schematic and State Diagram

- Student implementation is this lab focused on the data path part and the ALU part. In this section of the report, only the schematic of the two parts will be included.

Figure: Schematic of data path

- This is the schematic for data path. To implement BL instruction, here 2 multiplexer is added to the data path file.  BLmuxA and BLmuxB are the two MUXs.

- BLmuxA will select the address of the R14 when executing BL instruction. Other wife, it will select Instr[15:12] like the original version. Regfile, wa will receive the data from this mux.

- BLmuxB will select PCPlus4 when BL is called. This mux can save the PCPlus4 to R14. The result of this MUX is connected to wd of the regfile. When both MUXs are selected to d1, the PCPlus4 can be saved to R14 and the program can return to the stop point after finishing executing a branch.

Figure: ALU schematic

Figure: EOR of ALU

Figure: BIC of ALU

- EOR instruction uses 32 XOR gates which compare bit between SrcA and SrcB.

- BIC instruction uses 32 AND gates with one inverted input. The inverted input are the bits from SrcB. Since ALUResult to be SrcA & ~SrcB. The bits from SrcA and inverted SrcB should go through the AND gates.

Code

armv4.v

//__MAIN MODULES__

//full CPU

module armv4(input clk,reset,

input [31:0] ReadData,

input [31:0] Instr,

output [31:0] PC,

output MemWrite,

output [31:0] ALUResult,WriteData);

wire [3:0] ALUFlags, ALUControl;

wire RegWrite, ALUSrc, MemtoReg, PCSrc;

wire [1:0] RegSrc,ImmSrc;

controller c( .clk(clk), .reset(reset), .Instr(Instr[31:12]), .ALUFlags(ALUFlags), .RegSrc(RegSrc), .RegWrite(RegWrite), .ImmSrc(ImmSrc), .ALUSrc(ALUSrc), .ALUControl(ALUControl), .MemWrite(MemWrite), .MemtoReg(MemtoReg), .PCSrc(PCSrc) );

datapath dp(.clk(clk), .reset(reset), .RegSrc(RegSrc), .RegWrite(RegWrite), .ImmSrc(ImmSrc), .ALUSrc(ALUSrc), .ALUControl(ALUControl), .MemtoReg(MemtoReg), .PCSrc(PCSrc), .ALUFlags(ALUFlags), .PC(PC), .Instr(Instr), .ALUResult(ALUResult), .WriteData(WriteData), .ReadData(ReadData) );

endmodule

//datapath

module datapath(input clk,reset,

input [1:0] RegSrc,

input RegWrite,

input [1:0] ImmSrc,

input ALUSrc,

input [3:0] ALUControl,

input MemtoReg,

input PCSrc,

input [31:0] Instr,

input [31:0] ReadData,

output [3:0] ALUFlags,

output [31:0] PC,

output [31:0] ALUResult, WriteData);

wire [31:0] PCNext,PCPlus4,PCPlus8;

wire [31:0] ExtImm, SrcA, SrcB, Result;

wire [3:0] RA1, RA2;

wire [3:0] temp1;

wire [31:0] temp2;

//next PC logic

mux2 #(32) pcmux(.d0(PCPlus4), .d1(Result), .s(PCSrc), .y(PCNext) );

flopenr #(32) pcreg(.clk(clk), .reset(reset), .d(PCNext), .q(PC), .en(1'b1) );

adder #(32) pcadd1(.a(PC), .b(32'b100), .y(PCPlus4) );

adder #(32) pcadd2(.a(PCPlus4), .b(32'b100), .y(PCPlus8) );

//register file logic

mux2 #(4) ra1mux(.d0(Instr[19:16]), .d1(4'b1111), .s(RegSrc[0]), .y(RA1));

mux2 #(4) ra2mux(.d0(Instr[3:0]), .d1(Instr[15:12]), .s(RegSrc[1]), .y(RA2));

// regfile rf (.clk(clk), .we3(RegWrite), .ra1(RA1), .ra2(RA2), .wa3(Instr[15:12]), .wd3(Result), .r15(PCPlus8), .rd1(SrcA), .rd2(WriteData) );

mux2 #(4) BLmuxA(.d0(Instr[15:12]), .d1(4'b1110), .s(RegSrc[0] & RegSrc[1]), .y(temp1)); // Additional mux2 to set BL

mux2 #(32) BLmuxB (.d0(Result), .d1(PCPlus4), .s(RegSrc[0] & RegSrc[1]), .y(temp2));

regfile rf (.clk(clk), .we3(RegWrite), .ra1(RA1), .ra2(RA2), .wa3(temp1), .wd3(temp2), .r15(PCPlus8), .rd1(SrcA), .rd2(WriteData) );

mux2 #(32) resmux(.d0(ALUResult), .d1(ReadData), .s(MemtoReg), .y(Result));

extend ext( .Instr(Instr[23:0]), .ImmSrc(ImmSrc), .ExtImm(ExtImm) );

//ALU logic

mux2 #(32) srcbmux(.d0(WriteData), .d1(ExtImm), .s(ALUSrc), .y(SrcB));

alu alu (.SrcA(SrcA), .SrcB(SrcB), .ALUControl(ALUControl), .ALUResult(ALUResult), .ALUFlags(ALUFlags) );

endmodule

//ALU

module alu(input [31:0] SrcA, SrcB,

input [3:0] ALUControl,

output [31:0] ALUResult,

output [3:0] ALUFlags);

wire [32:0] condinvb;

wire [32:0] sum;

wire ofw1;

wire ofw0;

wire Cout;

wire add_op;        // if true, this is an add operation

wire sub_op;        // if true, this is a sub operation

wire add_sub_op;    // if true, this is an add or sub operation

// set internal signals

assign add_op =

(ALUControl == 4'b0100) || (ALUControl == 4'b1011);

assign sub_op =

(ALUControl == 4'b0010) || (ALUControl == 4'b1010);

assign add_sub_op = add_op | sub_op;

assign condinvb =

(sub_op == 1'b1) ? ~{1'b0,SrcB} : {1'b0,SrcB};

assign sum = {1'b0,SrcA} + condinvb + sub_op;

assign Cout = sum[32];

assign ofw1 = ~(SrcA[31] ^ SrcB[31] ^ sub_op);

assign ofw0 = SrcA[31] ^ SrcB[31] ^ add_sub_op;

assign ALUResult =

(ALUControl == 4'b0000) ? SrcA & SrcB : // AND

(ALUControl == 4'b1100) ? SrcA | SrcB : // OR

(ALUControl == 4'b0001) ? SrcA ^ SrcB : // EOR

(ALUControl == 4'b1110) ? SrcA & ~SrcB :// BIC

(ALUControl == 4'b1111) ? ~SrcB :       // MVN

(ALUControl == 4'b1000) ? SrcA & SrcB : // TST

(ALUControl == 4'b1001) ? SrcA ^ SrcB : // TEQ//(add_sub_op == 1'b1 && ALUControl != 4'b1010 && ALUControl != 4'b1011) ? sum[31:0] :

(add_sub_op == 1'b1) ? sum[31:0] :

// sub is 2's comp. same adder is used for add and sub

32'b0; // unrecognized ALU op code

//Set Flags

assign ALUFlags[3] =

(ALUControl == 4'b1000) ? ((SrcA & SrcB) >> 31) : // TST

(ALUControl == 4'b1001) ? ((SrcA ^ SrcB) >> 31) : // TEQ

sum[31]; //N negative

assign ALUFlags[2] =

(ALUControl == 4'b1000) ? ((SrcA & SrcB) == 32'b0) :

// TST

(ALUControl == 4'b1001) ? ((SrcA ^ SrcB) == 32'b0) :

// TEQ

(sum[31:0] == 32'b0); //Z zero

assign ALUFlags[1] = Cout & add_sub_op; // C carry-out

assign ALUFlags[0]= ofw1 & ofw0 & add_sub_op; //V overflow

// set final result

endmodule

//controller

module controller(input clk,reset,

input [31:12] Instr,

input [3:0] ALUFlags,

output [1:0] RegSrc,

output RegWrite,

output [1:0] ImmSrc,

output ALUSrc,

output [3:0] ALUControl,

output MemWrite, MemtoReg,

output PCSrc);

wire [1:0] FlagW;

wire PCS, RegW, MemW;

decoder dec( .Op(Instr[27:26]), .Funct(Instr[25:20]), .Rd(Instr[15:12]), .FlagW(FlagW), .PCS(PCS), .RegW(RegW), .MemW(MemW), .MemtoReg(MemtoReg), .ALUSrc(ALUSrc), .ImmSrc(ImmSrc), .RegSrc(RegSrc), .ALUControl(ALUControl) );

condlogic cl( .clk(clk), .reset(reset), .Cond(Instr[31:28]), .ALUFlags(ALUFlags), .FlagW(FlagW), .PCS(PCS), .RegW(RegW), .MemW(MemW), .PCSrc(PCSrc), .RegWrite(RegWrite), .MemWrite(MemWrite) );

endmodule

//condlogic

module condlogic(input clk, reset,

input  [3:0] Cond,

input  [3:0] ALUFlags,

input  [1:0] FlagW,

input  PCS, RegW, MemW,

output  PCSrc, RegWrite, MemWrite); // will be wires in tb

wire [1:0] FlagWrite;

wire [3:0] Flags;

//reg [3:0] Flags_r;

//assign Flags = Flags_r;

wire CondEx;

flopenr #(2) flagreg1( .clk(clk), .reset(reset), .en(FlagWrite[1]),.d(ALUFlags[3:2]), .q(Flags[3:2]) );

flopenr #(2) flagreg0( .clk(clk), .reset(reset), .en(FlagWrite[0]),.d(ALUFlags[1:0]), .q(Flags[1:0]) );

// write controls are conditional

condcheck cc(.Cond(Cond), .Flags(Flags), .CondEx(CondEx) );

assign FlagWrite = FlagW & {2{CondEx}};

assign RegWrite = RegW & CondEx;

assign MemWrite = MemW & CondEx;

assign PCSrc = PCS & CondEx;

endmodule

//condcheck

module condcheck(input  [3:0] Cond,

input [3:0] Flags,

output reg CondEx);

wire neg, zero, carry, overflow, ge;

assign {neg, zero, carry, overflow} = Flags;

assign ge = (neg == overflow);

always @(*)  begin

case(Cond)

4'b0000: CondEx = zero;             // EQ

4'b0001: CondEx = ~zero;            // NE

4'b0010: CondEx = carry;            // CS

4'b0011: CondEx = ~carry;           // CC

4'b0100: CondEx = neg;              // MI

4'b0101: CondEx = ~neg;             // PL

4'b0110: CondEx = overflow;         // VS

4'b0111: CondEx = ~overflow;        // VC

4'b1000: CondEx = carry & ~zero;    // HI

4'b1001: CondEx = ~(carry & ~zero); // LS

4'b1010: CondEx = ge;               // GE

4'b1011: CondEx = ~ge;              // LT

4'b1100: CondEx = ~zero & ge;       // GT

4'b1101: CondEx = ~(~zero & ge);    // LE

4'b1110: CondEx = 1'b1;             // Always

default: CondEx = 1'bx;             // undefined

endcase

end

endmodule

//decoder

module decoder(input  [1:0] Op,

input  [5:0] Funct,

input  [3:0] Rd,

output [1:0] FlagW,

output PCS, RegW, MemW,

output MemtoReg, ALUSrc,

output [1:0] ImmSrc, RegSrc,

output [3:0] ALUControl);

wire [9:0] controls;

wire Branch, ALUOp;

wire [1:0] FlagW_;

assign FlagW = FlagW_;

wire temp;

// Main Decoder

assign controls =

(Op == 2'b00) ? ((Funct[5]) ? 10'b0000101001 : 10'b0000001001) :

// Data-processing immediate operand : Data-processing register operand

(Op == 2'b01) ? ((Funct[0]) ? 10'b0001111000 : 10'b1001110100) :

// LDR : STR

//(Op == 2'b10) ? 10'b0110100010 : 10'b0;// B : undefined

(Op == 2'b10) ? ((Funct[0]) ? 10'b1110101010 : 10'b0110100010 ) : // BL : B

10'b0;

assign {RegSrc, ImmSrc, ALUSrc, MemtoReg, temp, MemW, Branch, ALUOp} = controls;

assign RegW = (Funct[4:1] == 4'b1000 || Funct[4:1] == 4'b1001 || Funct[4:1] == 4'b1010 || Funct[4:1] == 4'b1011 ) ? 0 : temp;

// ALU Decoder

wire add_sub_op;

assign add_sub_op =

(Funct[4:1] == 4'b0100 ||

Funct[4:1] == 4'b0010 ||

Funct[4:1] == 4'b1011 ||

Funct[4:1] == 4'b1010);

// this is a summation instruction

assign ALUControl = (ALUOp == 1'b1) ? Funct[4:1] : 4'b0100;

// if ALU instruction, set ALU control signals.

// Otherwise perform addition for Branch and memory target address calculation

assign FlagW_[1] = (ALUOp & Funct[0]);

// update ALU flags on ALU instruction completion if ALU op and S bit is set

assign FlagW_[0] = FlagW_[1] & add_sub_op;

// ALU C (carry) and V (overflow) flags are only updated if summation is being performed

// PC Logic

assign PCS = ((Rd == 4'b1111) & RegW) | Branch;

endmodule

//__GENERIC BUILDING BLOCKS__

//mux2

module mux2 #(parameter WIDTH = 8)

(input [WIDTH-1:0] d0,d1,

input s,

output [WIDTH-1:0] y);

assign y= s ? d1 : d0;

endmodule

//flopenr

module flopenr #(parameter WIDTH = 8)

(input clk,reset,en,

input [WIDTH-1:0] d,

output reg [WIDTH -1:0] q);

always @(posedge clk, posedge reset)begin

if (reset) q<=0;

else if (en) q<=d;

end

endmodule

//extend

module extend(input [23:0] Instr,

input [1:0] ImmSrc,

output reg [31:0] ExtImm);

always @(*) begin

case(ImmSrc)

2'b00: ExtImm={24'b0,Instr[7:0]};

//8bit unsigned imm

2'b01: ExtImm={20'b0,Instr[11:0]};

//12 bit unsignd imm

2'b10: ExtImm={ {6{Instr[23]}}, Instr[23:0], 2'b00 };

//24 bit 2s comp shifted branch

default: ExtImm=32'bx;

endcase

end

endmodule

//adder

module adder #(parameter WIDTH=8)

(input [WIDTH-1:0] a,b,

output [WIDTH-1:0] y);

assign y=a+b;

endmodule

// regfile

module regfile( input clk,

input we3,

input  [3:0] ra1, ra2, wa3,

input  [31:0] wd3, r15,

output [31:0] rd1, rd2);

reg [31:0] rf[14:0];

//3 ported register file

//read 2 ports combinationally

//write 3rd port on rising clock edge

//register 15 reads PC+8 instead

always @(posedge clk) begin

if (we3) rf[wa3]<=wd3;

end

assign rd1=(ra1==4'b1111) ? r15 : rf[ra1];

assign rd2=(ra2==4'b1111) ? r15 : rf[ra2];

endmodule

ARM_system.v

//Top module. Includes setting of clock

module ARM_system(input clk, reset,

output [31:0] PC,Instr,

output [31:0] WriteData, DataAdr,

output MemWrite);

reg [30:0]counter;

reg CLK1;

// clock divider

always @(posedge clk) begin

counter=counter+1;

if (counter==25000000) begin        // for FPGA

//if (counter==2) begin                 // for SIM

counter=0;

CLK1=!CLK1;

end

end

wire [31:0]  ReadData;

// instantiate processor and memories

armv4 cpu( .clk(CLK1), .reset(reset), .PC(PC), .Instr(Instr), .MemWrite(MemWrite), .ALUResult(DataAdr),.WriteData(WriteData), .ReadData(ReadData) );

imem imem( .clk(CLK1), .a(PC), .rd(Instr) );

dmem dmem( .clk(CLK1), .we(MemWrite), .a(DataAdr), .wd(WriteData), .rd(ReadData) );

endmodule

//Dmem module

module dmem(input clk, we,

input [31:0] a, wd,

output [31:0] rd);

reg [31:0] RAM[0:63];

assign rd = RAM[a[31:2]]; // word aligned

always @(posedge clk) begin

if (we) RAM[a[31:2]] <= wd;

end

endmodule

//Imem module

module imem(input clk,

input [31:0] a,

output reg [31:0] rd);

reg [31:0] RAM[0:63];

initial

//$readmemh("memfile.txt",RAM);

$readmemh("All_Inst (copy).txt", RAM);

always @(negedge clk) begin

rd = RAM[a[31:2]]; // word aligned

end

endmodule

All_Inst(copy).txt, ARM_system.v, and 7seg.v are not modified. These files are provided.

Additional Function

- EOR

For EOR instruction,  students used SrcA ^ SrcB to get the result. The result of this operation need to be saved. So students set the ALUResult and the function will take care of flags.

(ALUControl == 4'b0001) ? SrcA ^ SrcB : // EOR

Whent the ALUcontrol equals to 0001, the EOR will be operated.

- BIC

According to the provided material, BIC instruction makes the SrcA & ~SrcB. This instruction is similar to the EOR, the result need to be kept.  In case of ALUControl = 1110, the instruction will operate.

(ALUControl == 4'b1110) ? SrcA & ~SrcB :// BIC

- MVN

This instruction will invert the number of SrcB. Very similar to the previous 2 instruction, when ALUControl == 1111, SrcB will be inverted.

(ALUControl == 4'b1111) ? ~SrcB :       // MVN

- Decoder Change

For the instructions like TST, TEQ, CMP, CMN, they need to use the memory of the CPU. The result of these instructions will be saved to the register file. To present the written data , we need to set RegW in the decoder to 0.

Code modification:

assign {RegSrc, ImmSrc, ALUSrc, MemtoReg, temp, MemW, Branch, ALUOp} = controls;

assign RegW = (Funct[4:1] == 4'b1000 || Funct[4:1] == 4'b1001 || Funct[4:1] == 4'b1010 || Funct[4:1] == 4'b1011 ) ? 0 : temp;

- TST

TST instruction is a test instruction. This means that the result will not be saved. However, flags need to be saved which means the result of SrcA & SrcB will determine the flag change.

Since it is a bitwise operation, only N and Z flags need to be updated. For the sign of the result, it needs to shift the 31 bit of SrcA & SrcB right, because it is a 2’s complement numbers. This value is the MSB, the machine should determine which flag to update. MSB is 1, N flag to 1. MSB is 0, N flag to 0.

Then it needs to update the Z flag. When the result is 0, Z flag will be updated.

Lastly, RegW will be set to 0 to prevent writing to register file.

- TEQ

TEQ check the equivalence of SrcA and SrcB. It is the same as TST that no need to save result to register file. The only thing need to change is N and Z flags. RegW should be 0 to prevent writing to register file

The following code is for TEQ and TST:

assign ALUResult =

(ALUControl == 4'b0000) ? SrcA & SrcB : // AND

(ALUControl == 4'b1100) ? SrcA | SrcB : // OR

(ALUControl == 4'b0001) ? SrcA ^ SrcB : // EOR

(ALUControl == 4'b1110) ? SrcA & ~SrcB :// BIC

(ALUControl == 4'b1111) ? ~SrcB :       // MVN

(ALUControl == 4'b1000) ? SrcA & SrcB : // TST

(ALUControl == 4'b1001) ? SrcA ^ SrcB : // TEQ//(add_sub_op == 1'b1 && ALUControl != 4'b1010 && ALUControl != 4'b1011) ? sum[31:0] :

(add_sub_op == 1'b1) ? sum[31:0] :

// sub is 2's comp. same adder is used for add and sub

32'b0; // unrecognized ALU op code

//Set Flags

assign ALUFlags[3] =

(ALUControl == 4'b1000) ? ((SrcA & SrcB) >> 31) : // TST

(ALUControl == 4'b1001) ? ((SrcA ^ SrcB) >> 31) : // TEQ

sum[31]; //N negative

assign ALUFlags[2] =

(ALUControl == 4'b1000) ? ((SrcA & SrcB) == 32'b0) :

// TST

(ALUControl == 4'b1001) ? ((SrcA ^ SrcB) == 32'b0) :

// TEQ

(sum[31:0] == 32'b0); //Z zero

- CMP and CMN

These 2 test instructions involved plus and minus operation. This means CMP and CMN need to update the flags and check the result. Overflows and carriers need to be considered. To make the ALU know a add or subtract is operated. Student set add_sub_op in the decoder. After computation. The result is stored in ALUResult. After updating the flags, RegW will be set to 0 to prevent writing in the register.

Here is the Code for implement these instructions:

wire add_sub_op;

assign add_sub_op =

(Funct[4:1] == 4'b0100 ||

Funct[4:1] == 4'b0010 ||

Funct[4:1] == 4'b1011 ||

Funct[4:1] == 4'b1010);

- CMP and CMN

Bl instructions needs to write PCPLus4 to the R14 so that the program can go back to where the branch happens. Student add 2 MUXs to implement this function.

BLmuxA, can select the address of the R14, which is 1110, when RegSrc[0] and RegSrc[1] are high. This happens when the BL instruction is called. The second MUX, BLmuxB, can select the PCPlus4 when both variables are high. These 2 MUX will not interrupt other instructions to be operated.

Here it needs to write to the register file, so that the RegW should be high.

The following part is the change in code:

MUX added for BL:

mux2 #(4) BLmuxA(.d0(Instr[15:12]), .d1(4'b1110), .s(RegSrc[0] & RegSrc[1]), .y(temp1)); // Additional mux2 to set BL

mux2 #(32) BLmuxB (.d0(Result), .d1(PCPlus4), .s(RegSrc[0] & RegSrc[1]), .y(temp2));

BL change in Decoder:

// Main Decoder

assign controls =

(Op == 2'b00) ? ((Funct[5]) ? 10'b0000101001 : 10'b0000001001) :

// Data-processing immediate operand : Data-processing register operand

(Op == 2'b01) ? ((Funct[0]) ? 10'b0001111000 : 10'b1001110100) :

// LDR : STR

//(Op == 2'b10) ? 10'b0110100010 : 10'b0;// B : undefined

(Op == 2'b10) ? ((Funct[0]) ? 10'b1110101010 : 10'b0110100010 ) : // BL : B

10'b0;

assign {RegSrc, ImmSrc, ALUSrc, MemtoReg, temp, MemW, Branch, ALUOp} = controls;

assign RegW = (Funct[4:1] == 4'b1000 || Funct[4:1] == 4'b1001 || Funct[4:1] == 4'b1010 || Funct[4:1] == 4'b1011 ) ? 0 : temp;

Simulation Result

In this part, student will show the simulation result from the Quaturs simulation waveform.

- EOR

Figure: EOR simulation result

The PC for EOR is 0x0C, returned value is 0x34. The CPU worked as expectation.

- BIC

Figure: BIC simulation result

From the Provided file, we know the PC of BIC is 0x14, here memory change to 0x04, use the register R5. The result of this instruction is 0x04. It works as expected.

- MVN

Figure: MVN simulation result

For MVN instruction, PC is 0x40, the return value should be 0xFFFFFF3A. In the diagram, after 0x40, memory address changed to 0xFFFFFF3A, MVN works as expected, the result is saved in R7.

It is the same as PC = 0x44, the mem_addr = 0xFFFFFF0E, R8.

- TST

Figure: TST simulation result

When PC = 0x1C, mem_addr is 1, which means TST get a correct result from ALU.

- TEQ

Figure: TEQ simulation result

TEQ is runned when PC = 0x28. The displayed value should be 0.  The simulation result is the same as expected.

- CMP

Figure CMP simulation result

When PC = 0x34, CMP is operated. The displayed value should be D4 according to the provided running result. Here the mem_addr= 0xFFFFFFD4, which means the address is D4 and instruction worked as expected.

- CMN

Figure Simulation of CMN

When PC = 0x48, CMN is operated. The displayed value is 48. Here the mem_add = 0xFFFFFE48. The result is the same as expected.

- BL

Figure: BL simulation result

After the BL executed,  the program will return R14, then jump to command PC = 0xB4. Here the 0x6C will be saed to R14 so that it can be read by 0xB8. Then in instruction 0xB8, 0x6C will be loaded and jump to PC = 0x6C.

- PC in the green box

The following diagram includes the running result in the green box of the correct running result. From PC = 0x7C to 0xA8. CPU should generate correct mem_addr. For other instructions, the CPU should generate correct mem_write_data.

Figure: PC from 0x70 to 0x84

Figure: PC from 0x84 to 0x98

Figure: PC from 0x98 to 0xA8

FPGA Functionality

- Work on FPGA

To make the design worked on the FPGA, the clock source of the CPU need to be modified. The main idea is using the clock source on the FPGA board. The CLK will read the PIN_N5. This makes the CPU read the 10M Hz clock provided by the board.

Besides, to make the result can be checked by human eyes, the overflow of the clock should be changed. The change was made in the ARM_system.v. The overflow is changed to 50000000 when operating on the FPGA board. So that the instruction will change every 1 second.

Change in ARM_system.v:

// clock divider

always @(posedge clk) begin

counter=counter+1;

//if (counter==25000000) begin      // for FPGA

if (counter==2) begin                   // for SIM

counter=0;

CLK1=!CLK1;

end

end

- Simulation results

Here student will only include the Hex display of the board

Verification

The verification of the design is not difficult in this lab, since there is a correct running result provided in the lab, so student can check the Quaturs simulation with the correct result. And each parameter is included in the simulation waveform.

All the Verilog can be downloaded to the FPGA board, the board can worked as expected. The running result is the same as the provided result. So the design is complete.

Conclusion

In this lab student implement a ARM v4 CPU. FPGA can verify the design and check if each instruction can work as expected. It is a very complex design of a ARM CPU. However, the FPGA can still simulate it. The cost of verification of the design is small.